# Functional Specification

Helonix Water Tank Controller WT1.1 – Operating Modes and Protections

| Item | Details |
| --- | --- |
| Document version | 1.2 |
| Date | 26-09-2026 |
| Firmware reviewed | STM32 branch fix/testing-report-24-09-26; ESP-01 web app branch fix/device-sync |
| Status | Draft for client approval |
| Companion document | WT1.1_Test_Matrix_v1.2.xlsx (test cases for every rule) |

## How to review this document

Every rule has an ID (for example AUTO-04). Please tick Yes or No in the “Client OK?” column for each rule, and write any change in the “Client Comment” column (yellow cells).

Answer the open questions in the “Open points” section.

You can also select any text and use Word’s Review → New Comment.

Sign the “Client approval” section at the end. Any later change becomes a new version of this document.

When reporting a problem after approval, please quote the rule ID.

## Contents

Right-click and choose Update Field to show the table of contents.

# 1. Terms used

| Term | Meaning |
| --- | --- |
| Tank level | Read from 4 probes (25%, 50%, 75%, 100%). A level must be stable for 5 s before it is accepted. If a wireless tank transmitter is paired, its level is used and the last received value is kept during short signal gaps. |
| Tank full | Level reaches 100%. |
| Dry run sensor | Sensor that shows whether water is actually flowing from the source. |
| Dry run time | How long the motor runs as a test before the dry run sensor is checked (setting "Dry Run", default 2 min). |
| Testing gap | How long the motor waits after a failed dry run test before trying again (default 30 min). |
| Retry count | How many failed dry run tests are allowed before the motor waits for the next trigger (default 5; 0 = unlimited). |
| Ground water (G.W) | Sensor that shows whether ground-water supply is present. A change is accepted only after it has lasted 10 s. |
| On key / Off key | An event that is allowed to start / stop the motor in a mode. |
| Start delay | After power-up the motor never starts in the first 10 s. |

# 2. General behaviour

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| GEN-01 | Tank level comes from the 4 level probes; a new level is accepted only after it is stable for 5 s. | ☐ Yes ☐ No |  |
| GEN-02 | If a wireless tank transmitter is paired, its level is used; during a signal gap the last received level is kept. | ☐ Yes ☐ No |  |
| GEN-03 | After power-up the motor never starts during the first 10 s. | ☐ Yes ☐ No |  |
| GEN-04 | A ground water change (YES/NO) is accepted only after it has lasted 10 s. | ☐ Yes ☐ No |  |
| GEN-05 | All settings (limits, timer slots, twist, buzzer, countdown time, last mode) are kept through power cuts and firmware updates. | ☐ Yes ☐ No |  |
| GEN-06 | The clock keeps running during a power cut on its backup battery, and is not changed by firmware updates. | ☐ Yes ☐ No |  |
| GEN-07 | Relay 2 gives a 2 s pulse when the motor starts and relay 3 a 2 s pulse when it stops (for an external starter panel). | ☐ Yes ☐ No |  |

# 3. Device buttons

Buttons from left: 1 = Refill/Reset (red), 2 = Select "P" (yellow), 3 = Up, 4 = Down. Long press = hold 2 s.

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| BTN-01 | Button 1 short press: Refill start / stop. Long press: Manual on / off. | ☐ Yes ☐ No |  |
| BTN-02 | Button 2 short press: Auto on / Auto off (motor off). Long press: open the menu (Device Setup has no Power Restore item). | ☐ Yes ☐ No |  |
| BTN-03 | Button 3 short press: Timer mode on / off. Long press: Semi-Auto on / off. | ☐ Yes ☐ No |  |
| BTN-04 | Button 4 short press: Countdown start (saved time) / stop. Long press: edit the countdown time. | ☐ Yes ☐ No |  |
| BTN-05 | Twist mode is started and stopped from Menu > Twist Mode or from the app. | ☐ Yes ☐ No |  |
| BTN-06 | Quick taps are always registered; in menus and edit screens holding Up/Down repeats the step and speeds up after 3 s. On the dashboard holding a button never acts as repeated short presses. | ☐ Yes ☐ No |  |

# 4. Auto mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| AUTO-01 | Auto is started with button 2 or from the app. | ☐ Yes ☐ No |  |
| AUTO-02 | When no mode is running and the level falls to 50%, the device starts Auto by itself - unless the user switched Auto or Manual off with its button. | ☐ Yes ☐ No |  |
| AUTO-03 | Motor ON when the level is at or below 50% - every time, also after the tank has been filled (client decision DEC-08). | ☐ Yes ☐ No |  |
| AUTO-05 | Motor OFF when the tank is full (100%). | ☐ Yes ☐ No |  |
| AUTO-06 | Ground water detected (NO to YES): motor ON if the level is at or below 75%. Ground water lost does NOT stop the motor (client decision DEC-09). | ☐ Yes ☐ No |  |
| AUTO-08 | Dry run enabled: after the dry run time the sensor is checked; water found - motor keeps running until the tank is full. | ☐ Yes ☐ No |  |
| AUTO-09 | Dry run enabled, no water: motor OFF, waits the testing gap, then tries again. | ☐ Yes ☐ No |  |
| AUTO-10 | After the retry count is used up, the motor waits for the next on key (ground water detected, Refill button, or the user). | ☐ Yes ☐ No |  |
| AUTO-11 | Button 2 while Auto is running: motor OFF, screen shows "AUTO M:OFF". This still counts as Auto for power restore. | ☐ Yes ☐ No |  |
| AUTO-12 | Voltage, current and max run protections stop the motor (see Protections). | ☐ Yes ☐ No |  |
| AUTO-13 | Other on keys: Refill button (the tank is filled, then Auto continues), the testing gap ending after a dry run failure, and power restore (see PWR-02). | ☐ Yes ☐ No |  |

# 5. Manual mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| MAN-01 | Manual is started with a long press of button 1 or from the app. | ☐ Yes ☐ No |  |
| MAN-02 | In Manual only the user (button or app) can switch the motor ON. | ☐ Yes ☐ No |  |
| MAN-03 | Motor OFF by: user, voltage fault, over/under current, max run time, power off. | ☐ Yes ☐ No |  |
| MAN-04 | Tank full does NOT stop the motor in Manual; the tank-full buzzer sounds for 30 s if enabled. | ☐ Yes ☐ No |  |
| MAN-05 | Dry run and ground water are not used in Manual. | ☐ Yes ☐ No |  |
| MAN-06 | When a protection (voltage, current, max run) trips, the motor stops and the device stays in Manual - screen shows MANUAL with the motor off. Only the user's press starts it again; Auto does not start by itself. | ☐ Yes ☐ No |  |

# 6. Timer mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| TMR-01 | Up to 5 time slots, each with an ON time, OFF time and its own days of the week; a slot can be switched off. | ☐ Yes ☐ No |  |
| TMR-02 | Timer is started with button 3 or from the app (Save & Start Timer). The Refill button also starts the motor (see Refill). | ☐ Yes ☐ No |  |
| TMR-03 | Motor ON when the clock enters an active slot (right time and right day). | ☐ Yes ☐ No |  |
| TMR-04 | Motor OFF at the slot's OFF time. | ☐ Yes ☐ No |  |
| TMR-05 | Outside every slot the motor stays OFF; ground water and dry run are ignored. | ☐ Yes ☐ No |  |
| TMR-06 | Inside a slot, ground water detected: motor ON. Ground water lost does NOT stop the motor (client decision DEC-09). | ☐ Yes ☐ No |  |
| TMR-08 | Dry run enabled: same test / testing gap / retry count as Auto. | ☐ Yes ☐ No |  |
| TMR-09 | While running, if the dry run sensor shows no water, the motor stops and waits the testing gap. | ☐ Yes ☐ No |  |
| TMR-10 | Tank full: motor OFF; it starts again as soon as the level drops below 100% while the slot is active (see decision DEC-01). | ☐ Yes ☐ No |  |
| TMR-11 | A slot switched off in the app or on the device stays off after saving and after a reboot. | ☐ Yes ☐ No |  |

# 7. Twist mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| TWS-01 | Twist is started / stopped from Menu > Twist Mode or from the app. Settings: ON duration, OFF duration (minutes), optional start and stop time. | ☐ Yes ☐ No |  |
| TWS-02 | ON period: motor ON for the ON duration. OFF period: motor OFF for the OFF duration. Then repeats. | ☐ Yes ☐ No |  |
| TWS-03 | Twist runs only between its start and stop time; if both are equal it runs all day. | ☐ Yes ☐ No |  |
| TWS-04 | Dry run enabled: tested at the start of every ON period; water found - motor runs the rest of the ON period. | ☐ Yes ☐ No |  |
| TWS-05 | Dry run enabled, no water: motor OFF for the rest of that ON period; the OFF period follows and the next ON period tests again. Twist does not stop. | ☐ Yes ☐ No |  |
| TWS-06 | If the ON duration is shorter than the dry run time, the check is made at the end of the ON period. | ☐ Yes ☐ No |  |
| TWS-08 | Ground water is not used in Twist (its on key is its own time only). | ☐ Yes ☐ No |  |
| TWS-09 | Tank full: motor OFF; Twist restarts (with an ON period) only when the level is at or below 25%. | ☐ Yes ☐ No |  |
| TWS-10 | Twist settings are saved and kept after a reboot. | ☐ Yes ☐ No |  |
| TWS-11 | After a power cut: if twist was running and its end time has not passed, it continues the same ON or OFF period with the time it had left (saved every minute, so up to 1 min may be repeated). If the end time has passed, the motor stays off. | ☐ Yes ☐ No |  |

# 8. Semi-Auto mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| SEMI-01 | Semi-Auto is started with a long press of button 3 or from the app (only the user has the on key); it is a single run to fill the tank. | ☐ Yes ☐ No |  |
| SEMI-02 | Motor ON when Semi-Auto starts. | ☐ Yes ☐ No |  |
| SEMI-03 | Off keys: button 3 long press again, tank full (100%). The motor does not restart by itself. | ☐ Yes ☐ No |  |
| SEMI-04 | Dry run enabled, no water after the dry run time: motor OFF and Semi-Auto ends. | ☐ Yes ☐ No |  |
| SEMI-05 | Voltage / current faults and max run time stop the motor and end Semi-Auto. | ☐ Yes ☐ No |  |
| SEMI-06 | After a power cut: if the Semi-Auto motor was running, it runs again until the tank is full. | ☐ Yes ☐ No |  |

# 9. Countdown mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| CD-01 | Countdown runs the motor for a set time (1-180 min, default 10 min); started with button 4 or from the app. | ☐ Yes ☐ No |  |
| CD-02 | Motor OFF when the time ends or the user stops it (button 4 or app). However the countdown ends, the mode that was running before it (e.g. Auto, Timer, Twist, Manual, or Auto switched off with button 2) continues and is shown on the screen. | ☐ Yes ☐ No |  |
| CD-08 | While a countdown runs, the screen always shows the time left - also when it was started from the app or resumed after a power cut. | ☐ Yes ☐ No |  |
| CD-03 | Tank full (100%): the countdown ends and the previous mode continues. | ☐ Yes ☐ No |  |
| CD-04 | The time is set with a long press of button 4 (Down +1, Up -1, Select). It is saved and used by every later button 4 press; a time set in the app is also saved as the button time. | ☐ Yes ☐ No |  |
| CD-05 | Dry run enabled, no water after the dry run time: Countdown ends. | ☐ Yes ☐ No |  |
| CD-06 | Voltage and current faults and max run time end the countdown; the previous mode continues. | ☐ Yes ☐ No |  |
| CD-07 | After a power cut: a countdown that had not finished switches the motor on again for the time it had left (saved every minute, so up to 1 min may be repeated). When it ends, the mode that was running before it continues - this is also kept in memory. | ☐ Yes ☐ No |  |

# 10. Refill

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| REF-00 | Refill is not a mode: it is a motor start button usable in any mode. The screen shows "Refill" while it runs; afterwards the mode from before continues. | ☐ Yes ☐ No |  |
| REF-01 | Refill is started with a short press of button 1 or from the app; it is not started if the tank is already full. | ☐ Yes ☐ No |  |
| REF-02 | Tank full: Refill ends and the previous mode (e.g. Auto) continues. | ☐ Yes ☐ No |  |
| REF-03 | Refill is stopped by the user (button 1) and by max run time; the previous mode continues. | ☐ Yes ☐ No |  |
| REF-04 | Dry run enabled, no water: Refill ends and the previous mode continues. | ☐ Yes ☐ No |  |
| REF-05 | Voltage / current fault: Refill ends and the previous mode continues. | ☐ Yes ☐ No |  |
| REF-06 | Power cut during Refill: after power returns the device is in the mode from before Refill, and that mode's power-cut rule applies (e.g. Auto: motor starts). | ☐ Yes ☐ No |  |

# 11. Protections

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| PRT-01 | Low / high voltage (default 180 V / 280 V): voltage outside the limits for 3 s stops the motor; it is checked all the time, so a bad supply also prevents a start. Recovers after 10 s back in range (Manual stays off until the user starts it). | ☐ Yes ☐ No |  |
| PRT-02 | Over-load current (default 25 A): above the limit for 3 s stops the motor; the first 3 s after a start are ignored. Auto, Timer and Twist may restart after the testing gap. | ☐ Yes ☐ No |  |
| PRT-03 | Under-load current (default off): below the limit for 3 s stops the motor; Auto, Timer and Twist may restart after the testing gap. | ☐ Yes ☐ No |  |
| PRT-04 | Max run time (default 150 min) applies in every mode: after running this long the motor stops. Refill and Countdown end (previous mode continues), Semi-Auto ends, Manual stays shown with the motor off; Auto, Timer and Twist may run again after the testing gap. | ☐ Yes ☐ No |  |
| PRT-05 | A limit of 0 (or switched off in the app) disables that protection. | ☐ Yes ☐ No |  |
| PRT-06 | Voltage and current can be calibrated from the app against a meter reading; with no load the current shows 0.0 A. | ☐ Yes ☐ No |  |

# 12. After a power cut

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| PWR-01 | There is no Power Restore setting (removed on client decision DEC-10/11). After a power cut the device always returns to its last mode and follows these rules; the motor never starts in the first 10 s. | ☐ Yes ☐ No |  |
| PWR-02 | Auto (motor ON, or switched off with button 2): Auto, motor ON after 10 s; stays OFF if the tank is full. | ☐ Yes ☐ No |  |
| PWR-03 | Manual: Manual, motor OFF (power off is an off key for Manual; one long press of button 1 starts it). | ☐ Yes ☐ No |  |
| PWR-04 | Timer: Timer; the motor starts if the clock is inside an active slot. | ☐ Yes ☐ No |  |
| PWR-05 | Twist: continues with the time left if its end time has not passed, otherwise off (see TWS-11). | ☐ Yes ☐ No |  |
| PWR-06 | Countdown: motor on for the time left (CD-07). Semi-Auto: runs until tank full if its motor was running (SEMI-06). Refill: previous mode's rule (REF-06). | ☐ Yes ☐ No |  |

# 13. Buzzer

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| BUZ-01 | Pump on: beeps once per second while the motor runs (if enabled). | ☐ Yes ☐ No |  |
| BUZ-02 | Tank full: buzzer for 30 s (if enabled). | ☐ Yes ☐ No |  |
| BUZ-03 | Tank empty (level reaches 0%): buzzer for 15 s (if enabled). | ☐ Yes ☐ No |  |
| BUZ-04 | Each buzzer can be switched on/off separately; all are off by default. | ☐ Yes ☐ No |  |

# 14. Display

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| DSP-01 | Line 1: mode, motor state and level, e.g. "AUTO M:ON 50%". | ☐ Yes ☐ No |  |
| DSP-02 | Line 2: G.W:YES/NO, "DRY" when the dry run sensor shows no water, and the time; "TANK FULL" when full. | ☐ Yes ☐ No |  |
| DSP-03 | Page 2: voltage, current and day, or the active fault (voltage / overload / underload error). | ☐ Yes ☐ No |  |

# 15. Factory reset

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| FAC-01 | Factory reset (menu or app) sets: dry run disabled (2 min), testing gap 30 min, max run 150 min, retry count 5, low/high voltage 180/280 V, over-load 25 A, under-load disabled, buzzer all off, countdown 10 min, timer slots and twist cleared. | ☐ Yes ☐ No |  |

# 16. Web app (ESP-01)

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| APP-01 | Every app page (settings, timer, twist, countdown) shows the values currently stored on the device, not app defaults. | ☐ Yes ☐ No |  |
| APP-02 | If the device does not answer, the page says so and shows the last known values. | ☐ Yes ☐ No |  |
| APP-03 | Apply Settings saves on the device and the page reloads the values the device confirmed. | ☐ Yes ☐ No |  |
| APP-04 | Status (motor, level, mode) updates within about 3 s. | ☐ Yes ☐ No |  |
| APP-05 | Timer page sends all 5 slots; has Save & Start and Stop Timer. | ☐ Yes ☐ No |  |
| APP-06 | Twist page shows saved settings and running state; has Save & Start and Stop Twist; durations in minutes. | ☐ Yes ☐ No |  |
| APP-07 | Countdown page shows a countdown that is already running on the device. | ☐ Yes ☐ No |  |
| APP-08 | Settings page has separate buzzer switches (pump on, tank full, tank empty) and factory reset. It has no Power Restore setting. | ☐ Yes ☐ No |  |

# 17. Open points – client decision needed

These behaviours are implemented as described under “Current behaviour”. Please choose an option or describe what is required.

| ID | Topic | Current behaviour | Options | Client decision / comment |
| --- | --- | --- | --- | --- |
| DEC-01 | Timer after tank full | Now: motor restarts as soon as the level drops below 100% while the slot is active. Auto and Twist wait for 25%. | (a) Keep as now (b) Restart only at 25% or below |  |
| DEC-03 | Automatic Auto when idle | Now: when no mode is running and the level falls to 50%, the device starts Auto by itself (not when Auto or Manual was switched off with its button). | (a) Keep as now (b) Never start Auto by itself |  |
| DEC-04 | Retry count used up | Now: after all dry run retries fail, the motor waits for ground water or the user. | (a) Keep as now (b) Retry again after a longer gap - give time |  |
| DEC-05 | Current fault recovery | Now: after an over/under-current trip the motor may restart after the testing gap (default 30 min). | (a) Keep as now (b) Different time - give time (c) Only the user restarts |  |
| DEC-06 | Starter relays | Now: relay 2 = 2 s pulse at motor start, relay 3 = 2 s pulse at motor stop. | (a) Correct (b) Different wiring - describe |  |
| DEC-07 | Twist days | Now: the app lets you pick days for Twist but the device runs Twist every day. | (a) Every day is fine (b) Add day selection |  |

## Decisions already made

| ID | Topic | Client decision (implemented) |
| --- | --- | --- |
| DEC-02 | Manual and tank full | Manual does not stop at tank full (Key Story). |
| DEC-08 | Auto start level | (b) Always 50%. |
| DEC-09 | Ground water lost | (b) Removed - ground water only starts the motor. |
| DEC-10 | Resume after a power cut | Power Restore setting removed; always follow the Key Story. |
| DEC-11 | Refill + power cut, previous mode Auto | Power Restore removed; (b) Auto rule - motor starts. |

# 18. Revision history

| Version | Date | Changes | By |
| --- | --- | --- | --- |
| 1.0 | 26-09-2026 | First issue for client approval. Includes the fixes from testing report 24-09-26. | Development |
| 1.1 | 26-09-2026 | Updated to the client's "Key Story" (on/off keys per mode): Auto starts below 50% (25%); Refill is not a mode and returns to the previous mode (also after a fault or power cut); Manual stays shown after a protection trip; Semi-Auto, Countdown and Twist continue after a power cut; Countdown ends at tank full and at max run and returns to the previous mode; Timer starts after a power cut inside an active slot. New open points DEC-08 to DEC-11. | Development |
| 1.2 | 26-09-2026 | Client decisions on DEC-08 to DEC-11: Auto always starts at 50%; ground water lost no longer stops the motor (ground water only starts it); the Power Restore setting is removed - after a power cut the last mode is always restored as per the Key Story (Semi-Auto until tank full, Countdown for the time left, Twist for the remaining duration if its end time has not passed). Ground water is not used in Twist. | Development |

# 19. Client approval

By signing, the client confirms that the rules in this document, together with the comments and decisions written in it, describe the required behaviour.

| Item | Client |
| --- | --- |
| Overall comments |  |
| Approved (Yes / Yes with changes / No) |  |
| Name |  |
| Designation / Company |  |
| Signature |  |
| Date |  |