# Functional Specification

Helonix Water Tank Controller WT1.1 – Operating Modes and Protections

| Item | Details |
| --- | --- |
| Document version | 1.0 |
| Date | 26-09-2026 |
| Firmware reviewed | STM32 branch fix/testing-report-24-09-26; ESP-01 web app branch fix/device-sync |
| Status | Draft for client approval |
| Companion document | WT1.1_Test_Matrix_v1.0.xlsx (test cases for every rule) |

## How to review this document

Every rule has an ID (for example AUTO-04). Please tick Yes or No in the “Client OK?” column for each rule, and write any change in the “Client Comment” column (yellow cells).

Answer the open questions in the “Open points” section.

You can also select any text and use Word’s Review → New Comment.

Sign the “Client approval” section at the end. Any later change becomes a new version of this document.

When reporting a problem after approval, please quote the rule ID.

## Contents

Right-click and choose Update Field to show the table of contents.

# 1. Terms used

| Term | Meaning |
| --- | --- |
| Tank level | Read from 4 probes (25%, 50%, 75%, 100%). A level must be stable for 5 s before it is accepted. If a wireless tank transmitter is paired, its level is used and the last received value is kept during short signal gaps. |
| Tank full | Level reaches 100%. |
| Dry run sensor | Sensor that shows whether water is actually flowing from the source. |
| Dry run time | How long the motor runs as a test before the dry run sensor is checked (setting "Dry Run", default 2 min). |
| Testing gap | How long the motor waits after a failed dry run test before trying again (default 30 min). |
| Retry count | How many failed dry run tests are allowed before the motor waits for the next trigger (default 5; 0 = unlimited). |
| Ground water (G.W) | Sensor that shows whether ground-water supply is present. A change is accepted only after it has lasted 10 s. |
| On key / Off key | An event that is allowed to start / stop the motor in a mode. |
| Start delay | After power-up the motor never starts in the first 10 s. |

# 2. General behaviour

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| GEN-01 | Tank level comes from the 4 level probes; a new level is accepted only after it is stable for 5 s. | ☐ Yes ☐ No |  |
| GEN-02 | If a wireless tank transmitter is paired, its level is used; during a signal gap the last received level is kept. | ☐ Yes ☐ No |  |
| GEN-03 | After power-up the motor never starts during the first 10 s. | ☐ Yes ☐ No |  |
| GEN-04 | A ground water change (YES/NO) is accepted only after it has lasted 10 s. | ☐ Yes ☐ No |  |
| GEN-05 | All settings (limits, timer slots, twist, buzzer, countdown time, power restore, last mode) are kept through power cuts and firmware updates. | ☐ Yes ☐ No |  |
| GEN-06 | The clock keeps running during a power cut on its backup battery, and is not changed by firmware updates. | ☐ Yes ☐ No |  |
| GEN-07 | Relay 2 gives a 2 s pulse when the motor starts and relay 3 a 2 s pulse when it stops (for an external starter panel). | ☐ Yes ☐ No |  |

# 3. Device buttons

Buttons from left: 1 = Refill/Reset (red), 2 = Select "P" (yellow), 3 = Up, 4 = Down. Long press = hold 2 s.

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| BTN-01 | Button 1 short press: Refill start / stop. Long press: Manual on / off. | ☐ Yes ☐ No |  |
| BTN-02 | Button 2 short press: Auto on / Auto off (motor off). Long press: open the menu. | ☐ Yes ☐ No |  |
| BTN-03 | Button 3 short press: Timer mode on / off. Long press: Semi-Auto on / off. | ☐ Yes ☐ No |  |
| BTN-04 | Button 4 short press: Countdown start (saved time) / stop. Long press: edit the countdown time. | ☐ Yes ☐ No |  |
| BTN-05 | Twist mode is started and stopped from Menu > Twist Mode or from the app. | ☐ Yes ☐ No |  |
| BTN-06 | Quick taps are always registered; in menus and edit screens holding Up/Down repeats the step and speeds up after 3 s. On the dashboard holding a button never acts as repeated short presses. | ☐ Yes ☐ No |  |

# 4. Auto mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| AUTO-01 | Auto is started with button 2 or from the app. | ☐ Yes ☐ No |  |
| AUTO-02 | When no mode is running and the level falls to 50%, the device starts Auto by itself - unless the user switched Auto off. | ☐ Yes ☐ No |  |
| AUTO-03 | Motor ON when the level is at or below 50% (first start). | ☐ Yes ☐ No |  |
| AUTO-04 | After Auto has filled the tank, the motor starts again only when the level is at or below 25% (stays off at 75% and 50%). | ☐ Yes ☐ No |  |
| AUTO-05 | Motor OFF when the tank is full (100%). | ☐ Yes ☐ No |  |
| AUTO-06 | Ground water detected (NO to YES): motor ON if the level is at or below 75%. | ☐ Yes ☐ No |  |
| AUTO-07 | Ground water lost (YES to NO) while running: motor OFF, then waits the testing gap. | ☐ Yes ☐ No |  |
| AUTO-08 | Dry run enabled: after the dry run time the sensor is checked; water found - motor keeps running until the tank is full. | ☐ Yes ☐ No |  |
| AUTO-09 | Dry run enabled, no water: motor OFF, waits the testing gap, then tries again. | ☐ Yes ☐ No |  |
| AUTO-10 | After the retry count is used up, the motor waits for the next on key (ground water detected, or the user). | ☐ Yes ☐ No |  |
| AUTO-11 | Button 2 while Auto is running: motor OFF, screen shows "AUTO M:OFF". This still counts as Auto for power restore. | ☐ Yes ☐ No |  |
| AUTO-12 | Voltage, current and max run protections stop the motor (see Protections). | ☐ Yes ☐ No |  |

# 5. Manual mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| MAN-01 | Manual is started with a long press of button 1 or from the app. | ☐ Yes ☐ No |  |
| MAN-02 | In Manual only the user (button or app) can switch the motor ON. | ☐ Yes ☐ No |  |
| MAN-03 | Motor OFF by: user, voltage fault, over/under current, max run time, power off. | ☐ Yes ☐ No |  |
| MAN-04 | Tank full does NOT stop the motor in Manual; the tank-full buzzer sounds for 30 s if enabled. | ☐ Yes ☐ No |  |
| MAN-05 | Dry run and ground water are not used in Manual. | ☐ Yes ☐ No |  |
| MAN-06 | When a protection trips, the motor stops and Manual mode ends; only the user can start it again. | ☐ Yes ☐ No |  |

# 6. Timer mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| TMR-01 | Up to 5 time slots, each with an ON time, OFF time and its own days of the week; a slot can be switched off. | ☐ Yes ☐ No |  |
| TMR-02 | Timer is started with button 3 or from the app (Save & Start Timer). | ☐ Yes ☐ No |  |
| TMR-03 | Motor ON when the clock enters an active slot (right time and right day). | ☐ Yes ☐ No |  |
| TMR-04 | Motor OFF at the slot's OFF time. | ☐ Yes ☐ No |  |
| TMR-05 | Outside every slot the motor stays OFF; ground water and dry run are ignored. | ☐ Yes ☐ No |  |
| TMR-06 | Inside a slot, ground water detected: motor ON. | ☐ Yes ☐ No |  |
| TMR-07 | Inside a slot, ground water lost: motor OFF, then waits the testing gap. | ☐ Yes ☐ No |  |
| TMR-08 | Dry run enabled: same test / testing gap / retry count as Auto. | ☐ Yes ☐ No |  |
| TMR-09 | While running, if the dry run sensor shows no water, the motor stops and waits the testing gap. | ☐ Yes ☐ No |  |
| TMR-10 | Tank full: motor OFF; it starts again as soon as the level drops below 100% while the slot is active (see decision DEC-01). | ☐ Yes ☐ No |  |
| TMR-11 | A slot switched off in the app or on the device stays off after saving and after a reboot. | ☐ Yes ☐ No |  |

# 7. Twist mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| TWS-01 | Twist is started / stopped from Menu > Twist Mode or from the app. Settings: ON duration, OFF duration (minutes), optional start and stop time. | ☐ Yes ☐ No |  |
| TWS-02 | ON period: motor ON for the ON duration. OFF period: motor OFF for the OFF duration. Then repeats. | ☐ Yes ☐ No |  |
| TWS-03 | Twist runs only between its start and stop time; if both are equal it runs all day. | ☐ Yes ☐ No |  |
| TWS-04 | Dry run enabled: tested at the start of every ON period; water found - motor runs the rest of the ON period. | ☐ Yes ☐ No |  |
| TWS-05 | Dry run enabled, no water: motor OFF for the rest of that ON period; the OFF period follows and the next ON period tests again. Twist does not stop. | ☐ Yes ☐ No |  |
| TWS-06 | If the ON duration is shorter than the dry run time, the check is made at the end of the ON period. | ☐ Yes ☐ No |  |
| TWS-07 | Ground water lost (YES to NO): motor OFF until ground water returns, then a new ON period starts. | ☐ Yes ☐ No |  |
| TWS-08 | If the ground water probe is not fitted (always NO), Twist ignores it. | ☐ Yes ☐ No |  |
| TWS-09 | Tank full: motor OFF; Twist restarts (with an ON period) only when the level is at or below 25%. | ☐ Yes ☐ No |  |
| TWS-10 | Twist settings are saved and kept after a reboot. | ☐ Yes ☐ No |  |

# 8. Semi-Auto mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| SEMI-01 | Semi-Auto is started with a long press of button 3 or from the app; it is a single run to fill the tank. | ☐ Yes ☐ No |  |
| SEMI-02 | Motor ON when Semi-Auto starts. | ☐ Yes ☐ No |  |
| SEMI-03 | Tank full: motor OFF and does not restart by itself. | ☐ Yes ☐ No |  |
| SEMI-04 | Dry run enabled, no water after the dry run time: motor OFF and Semi-Auto ends. | ☐ Yes ☐ No |  |
| SEMI-05 | Protections stop the motor; max run time ends Semi-Auto. | ☐ Yes ☐ No |  |

# 9. Countdown mode

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| CD-01 | Countdown runs the motor for a set time (1-180 min, default 10 min); started with button 4 or from the app. | ☐ Yes ☐ No |  |
| CD-02 | Motor OFF when the time ends or the user stops it (button 4 or app). However the countdown ends, the mode that was running before it (e.g. Auto, Timer, Twist, Manual, or Auto switched off with button 2) continues and is shown on the screen. | ☐ Yes ☐ No |  |
| CD-08 | While a countdown runs, the screen always shows the time left - also when it was started from the app or resumed after a power cut. | ☐ Yes ☐ No |  |
| CD-03 | Tank full: motor OFF and stays off until the time ends. | ☐ Yes ☐ No |  |
| CD-04 | The time is set with a long press of button 4 (Down +1, Up -1, Select). It is saved and used by every later button 4 press; a time set in the app is also saved as the button time. | ☐ Yes ☐ No |  |
| CD-05 | Dry run enabled, no water after the dry run time: Countdown ends. | ☐ Yes ☐ No |  |
| CD-06 | Voltage and current faults end the Countdown. Max run time does not apply. | ☐ Yes ☐ No |  |
| CD-07 | Power restore ON: a countdown running at power loss continues with the time it had left (saved every minute, so up to 1 min may be repeated). When it ends, the mode that was running before it continues - this is also kept in memory. | ☐ Yes ☐ No |  |

# 10. Refill

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| REF-01 | Refill is started with a short press of button 1 or from the app; it is not started if the tank is already full. | ☐ Yes ☐ No |  |
| REF-02 | Tank full: Refill ends and the previous mode (e.g. Auto) continues. | ☐ Yes ☐ No |  |
| REF-03 | Refill is stopped by the user (button 1), max run time or protections. | ☐ Yes ☐ No |  |
| REF-04 | Dry run enabled, no water: Refill ends and the previous mode continues. | ☐ Yes ☐ No |  |

# 11. Protections

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| PRT-01 | Low / high voltage (default 180 V / 280 V): voltage outside the limits for 3 s stops the motor; it is checked all the time, so a bad supply also prevents a start. Recovers after 10 s back in range; Manual ends. | ☐ Yes ☐ No |  |
| PRT-02 | Over-load current (default 25 A): above the limit for 3 s stops the motor; the first 3 s after a start are ignored. Motor may restart after the testing gap; Manual ends. | ☐ Yes ☐ No |  |
| PRT-03 | Under-load current (default off): below the limit for 3 s stops the motor; restart after the testing gap; Manual ends. | ☐ Yes ☐ No |  |
| PRT-04 | Max run time (default 150 min): after running this long the motor stops. Manual, Semi-Auto and Refill end; Auto, Timer and Twist may run again after the testing gap; not applied to Countdown. | ☐ Yes ☐ No |  |
| PRT-05 | A limit of 0 (or switched off in the app) disables that protection. | ☐ Yes ☐ No |  |
| PRT-06 | Voltage and current can be calibrated from the app against a meter reading; with no load the current shows 0.0 A. | ☐ Yes ☐ No |  |

# 12. Power restore

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| PWR-01 | Device Setup > PwrRestore has two values: ON and OFF (factory default OFF). | ☐ Yes ☐ No |  |
| PWR-02 | ON, last mode Auto (motor ON, or switched off with button 2): after power returns - Auto, motor ON after 10 s; stays OFF if the tank is full. | ☐ Yes ☐ No |  |
| PWR-03 | ON, last mode Manual: Manual, motor OFF (one long press of button 1 starts it). | ☐ Yes ☐ No |  |
| PWR-04 | ON, last mode Timer: Timer; motor runs again if it was running and the slot is still active. | ☐ Yes ☐ No |  |
| PWR-05 | ON, last mode Twist: Twist continues with an ON period if the motor was running, else with an OFF period. | ☐ Yes ☐ No |  |
| PWR-06 | ON, last mode Semi-Auto / Refill: not restored (one-time runs). Countdown: continues with the time it had left (see CD-07). | ☐ Yes ☐ No |  |
| PWR-07 | OFF: after power returns always Manual, motor OFF. | ☐ Yes ☐ No |  |

# 13. Buzzer

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| BUZ-01 | Pump on: beeps once per second while the motor runs (if enabled). | ☐ Yes ☐ No |  |
| BUZ-02 | Tank full: buzzer for 30 s (if enabled). | ☐ Yes ☐ No |  |
| BUZ-03 | Tank empty (level reaches 0%): buzzer for 15 s (if enabled). | ☐ Yes ☐ No |  |
| BUZ-04 | Each buzzer can be switched on/off separately; all are off by default. | ☐ Yes ☐ No |  |

# 14. Display

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| DSP-01 | Line 1: mode, motor state and level, e.g. "AUTO M:ON 50%". | ☐ Yes ☐ No |  |
| DSP-02 | Line 2: G.W:YES/NO, "DRY" when the dry run sensor shows no water, and the time; "TANK FULL" when full. | ☐ Yes ☐ No |  |
| DSP-03 | Page 2: voltage, current and day, or the active fault (voltage / overload / underload error). | ☐ Yes ☐ No |  |

# 15. Factory reset

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| FAC-01 | Factory reset (menu or app) sets: dry run disabled (2 min), testing gap 30 min, max run 150 min, retry count 5, low/high voltage 180/280 V, over-load 25 A, under-load disabled, power restore OFF, buzzer all off, countdown 10 min, timer slots and twist cleared. | ☐ Yes ☐ No |  |

# 16. Web app (ESP-01)

| ID | Rule | Client OK? | Client Comment |
| --- | --- | --- | --- |
| APP-01 | Every app page (settings, timer, twist, countdown) shows the values currently stored on the device, not app defaults. | ☐ Yes ☐ No |  |
| APP-02 | If the device does not answer, the page says so and shows the last known values. | ☐ Yes ☐ No |  |
| APP-03 | Apply Settings saves on the device and the page reloads the values the device confirmed. | ☐ Yes ☐ No |  |
| APP-04 | Status (motor, level, mode) updates within about 3 s. | ☐ Yes ☐ No |  |
| APP-05 | Timer page sends all 5 slots; has Save & Start and Stop Timer. | ☐ Yes ☐ No |  |
| APP-06 | Twist page shows saved settings and running state; has Save & Start and Stop Twist; durations in minutes. | ☐ Yes ☐ No |  |
| APP-07 | Countdown page shows a countdown that is already running on the device. | ☐ Yes ☐ No |  |
| APP-08 | Settings page has separate buzzer switches (pump on, tank full, tank empty), power restore ON/OFF and factory reset. | ☐ Yes ☐ No |  |

# 17. Open points – client decision needed

These behaviours are implemented as described under “Current behaviour”. Please choose an option or describe what is required.

| ID | Topic | Current behaviour | Options | Client decision / comment |
| --- | --- | --- | --- | --- |
| DEC-01 | Timer after tank full | Now: motor restarts as soon as the level drops below 100% while the slot is active. Auto and Twist wait for 25%. | (a) Keep as now (b) Restart only at 25% or below |  |
| DEC-02 | Manual and tank full | Now: Manual does not stop at tank full (tank full is not an off key for Manual in the testing report). | (a) Keep as now (b) Stop Manual at tank full |  |
| DEC-03 | Automatic Auto when idle | Now: when no mode is running and the level falls to 50%, the device starts Auto by itself (unless the user switched Auto off). | (a) Keep as now (b) Never start Auto by itself |  |
| DEC-04 | Retry count used up | Now: after all dry run retries fail, the motor waits for ground water or the user. | (a) Keep as now (b) Retry again after a longer gap - give time |  |
| DEC-05 | Current fault recovery | Now: after an over/under-current trip the motor may restart after the testing gap (default 30 min). | (a) Keep as now (b) Different time - give time (c) Only the user restarts |  |
| DEC-06 | Starter relays | Now: relay 2 = 2 s pulse at motor start, relay 3 = 2 s pulse at motor stop. | (a) Correct (b) Different wiring - describe |  |
| DEC-07 | Twist days | Now: the app lets you pick days for Twist but the device runs Twist every day. | (a) Every day is fine (b) Add day selection |  |

# 18. Revision history

| Version | Date | Changes | By |
| --- | --- | --- | --- |
| 1.0 | 26-09-2026 | First issue for client approval. Includes the fixes from testing report 24-09-26. | Development |

# 19. Client approval

By signing, the client confirms that the rules in this document, together with the comments and decisions written in it, describe the required behaviour.

| Item | Client |
| --- | --- |
| Overall comments |  |
| Approved (Yes / Yes with changes / No) |  |
| Name |  |
| Designation / Company |  |
| Signature |  |
| Date |  |